Budyko Calibration

PB 28/06/2024

# Hypotheses:

Budyko calibration results will be not as good fitting simulated to the observed discharge as if it is calibrated for discharge itself, but it will be an improvement against an unfitted a priori parameter run.

Water

Energy

Dryness index

Potential evaporation / precipitation

Evaporative index

Actual evaporation / precipitation

Advantage:

Precipitation, and evaporation is available everywhere

Fig1: Budyko function as relation between Eact/Precipitation and Epot/Precipitation

# Method

It uses the  same scripts as for discharge calibration, but objective function is changed.

Instead maximize KGE (or NSE) for discharge

The distance to the optimal” Budyko function is minimized

Fig 2: Budyko points of a basin at initial run

Fig 3: Budyko points of a basin at initial run and best Budyko run

If it works out, the Budyko calibration show better results than the Zero run, but worse than the discharge calibration.

Fig 4: Discharge for observed, zero run and Budyko run.

For discharge calibration parameters are calibrated for each hydrological process – snow, evaporation, soil, groundwater, routing, lakes.

Because the Budyko curve looks at runoff generation (and evaporation) at grid cell level the runoff concentration and the routing processes are not sensitive to the objective function of the Budyko calibration. Therefore less parameters can are calibrated.

Budyko points:

For each grid cell the sum of daily precipitation (P), potential evaporation (ETP) and actual evapotranspiration (ETA) is calculated. From these three sums the coordinate in the “Budyko space” are calculated:

x= ETPP;y= ETAP

Depending on the period of calibration, the sum is calculated for 10 to 15 years.

The “Budyko space” spanned by x,y for each grid cell should be close to the Budyko curve:

y=1+x-1+xω1ω          with fixed ω = 2.6.

Here, the distance of Kolmogorov-Smirnov (maximum distance of a point to the function) is used as objective function and the calibration algorithm minimizes this distance.

# How to implement

- Get the sum of precipitation, potential and actual evaporation for each grid cell for the whole time period.

In CWatM we have a defined output variable: 
OUT_TSS_TotalTot = totalET, Precipitation, ETRef

Fig 5: Timeseries output for sum of precipitation for each grid cell

In Lisflood you might do some additional processing to calculate the results (I did not check yet, what to do)

- In the calibration tools replace the KGE call:

p_csv = os.path.join(directory_run, "Precipitation_totaltot.txt")
etp_csv = os.path.join(directory_run, "ETRef_totaltot.txt")

Prec1 = pandas.read_table(p_csv, sep=r"\s+", index_col=0, skiprows=3, header=None, skipinitialspace=True)
ETP1 = pandas.read_table(etp_csv, sep=r"\s+", index_col=0, skiprows=3, header=None, skipinitialspace=True)
Prec = Prec1.index.values
ETP = ETP1.index.values
BudykoX = ETP / Prec

Precipition and ETRef is for all runs the same. Needs to be done only once

Etact_tss = 'totalET_totaltot.txt'

Etact1 = pandas.read_table(etact_tss, sep=r"\s+", index_col=0, skiprows=3, header=None, skipinitialspace=True)
Etact = Etact1.index.values

BudykoY = Etact / Prec

This is the output of each modelrun.

And then it goes to hydrostat subroutine and is returned instead of KGE

budyko = hydroStats.Budyko(BudykoX, BudykoY)

return budyko

- Hydrostat submodule

The hydrostat submodule evaluates the observed and simulated timeseries and returns objective functions. Here, we added a few functions to return a Budyko objective function.

Here:  the distance of Kolmogorov-Smirnov (maximum distance of a point to the function)

- Changing the DEAP parameters

Instead of maximizing the KGE value (to get as close as possible to 1)

you minimize the value.

Change this in the settings file.

[DEAP]

maximize = False

# Results

Fig 5

Fig 7. Comparison of KGE for Zero, Budyko and Discharge calibration. 5 armin CWatM model for 1723 stations. Monthly discharge values.

Figure 7 shows that Budyko calibration generally improves calibration especially for zero runs below 0.5 KGE. The less successful the zero run is, the better the Budyko calibration.